Notes on Power Estimation using IRSIM

======================================

New commands		Descriptions

------------------------------------------------------------------------------

powlogfile		start/stop power logfile

powtrace		start/stop power tracing of specified node(s)/vector(s)

powstep		toggle the display of power estimate for each timestep

sumcap		print out the sum of the capacitance of all nodes

vsupply		set supply voltage for calculating power (default 5V)

------------------------------------------------------------------------------

Example usage of new commands:

irsim> powlogfile foo.trace

Opens foo.trace to which nodal transition reports will

contain nodal capacitance data (n->ncap from the node structure).

The format of each line is similar to the line you get when you trace a

node normally:

-------------------------

8300.0 *INPUT* ir/ICacheDataBus_w2.5 -> 0       0.0 0.0 0.390

for an input being changed

-------------------------

/m/s[25]/bitbar        0 -> 1  0.8 0.8 0.494

for a normal node change

-------------------------

The last field is the node capacitance being switched (in pF)

Note that this file can be useful if you want to catch transitions

yourself and process them.  Ordinarily, I do the following:

irsim> powlogfile /dev/null

which throws away the transitional data.  However, upon closing the "file",

a summary will come to stdout as shown below.

irsim> powlogfile

With no arguments, powlogfile just closes the opened logfile.

irsim> powtrace <node names>

'powtrace' is used to indicate to irsim which nodes to log power statistics

on.  For instance:

irsim> powtrace *

will log all transitions to the powlog file, and the summary that is

displayed upon closing the file will contain data for all nodes.

Following the above command with a line like the following:

irsim> powtrace -foo/dont/count/me*

would mean that all nodes with the prefix 'foo/dont/count/me' would

not be included in the powlog file or in the summary.  Note that

this is the same syntax for the 't' (trace) command that already exists

in irsim.

Instead of tracing all nodes by using *, one can look at only the nodes

that are interesting by using lines of the form:

irsim> powtrace BlockA/interesting/nodes*

irsim> powtrace BlockB/moreInteresting/nodes*

again, the same syntax for the 't' command.

'powtrace' also sets the context for the 'powstep' command:

irsim> powstep

toggles whether dynamic power estimation is displayed after each

timestep.

Again, the dynamic power displayed will only be for the nodes that

have been selected using the 'powtrace' command.

irsim> vsupply 5.0

Supply Voltage = 5.00 Volts

Sets V=5.0 for use in the P=CV^2/(2t) where C is capacitance switched,

and t is the timestep.  The default value for vsupply is 5.0 Volts.

irsim> sumcap

Sum of nodal capacitances: 3699.549805 pF

Gives a sum of ALL nodal capcitances, NOT just those selected with

the 'powtrace' command.

===========================================================================

Description of the power summary that is printed when the powlogfile is

closed:

For each node selected using 'powtrace' a line will be printed to stdout

in the following format:

e/E/c/Phi/5_498_60       2.486        225            0.154719       0.005565

name                    cap (pF)  # of transitions   mWatts         % of total

(see note below)		     dissipated in  power due

this node      to this

node

Note:  Nodal transitions caused by 'h <node>' or 'l <node>' are

not included in the transition count.

After all the node lines go by, a final line will be printed giving the

total power dissipation of the traced nodes in Watts.

============================================================================

Example Script:

| This is a script that shows what I usually do:

|

| Normal initialization

@init.cmd

| Throw away transitional data

powlogfile /dev/null

| Display power at each step

powstep

| Invoke simulation script that prods all the right nodes at the right time

@simulate.cmd

| Open a normal logfile to catch the summary table

logfile /tmp/power.report

| Close the powlogfile to generate the report

powlogfile

| Close the logfile

logfile

=============================================================================

Now, with grep, awk, sort, and other great programs you might write, you can

pull whatever data you want out of the resulting files.

Typing

irsim> help [command]

should give you a help message for the new commands just like it does for

the old ones

That's all, folks....

If you find bugs or have questions or suggestions, please let me know by

sending mail to dcr@fugue.